// ==========================================

// ColdChain Sentinel Prototype

// ESP32 + DHT11 + MQ135 + LCD + Relays

// ==========================================

// | Component   | GPIO    |

// | ----------- | ------- |

// | DHT11 Room1 | GPIO 4  |

// | DHT11 Room2 | GPIO 5  |

// | MQ135       | GPIO 34 |

// | Relay 1     | GPIO 18 |

// | Relay 2     | GPIO 19 |

// | Relay 3     | GPIO 21 |

// | Red LED     | GPIO 22 |

// | Green LED   | GPIO 23 |

// | LCD SDA     | GPIO 26 |

// | LCD SCL     | GPIO 27 |

#include <WiFi.h>

#include <HTTPClient.h>

#include <ArduinoJson.h>

#include <DHT.h>

#include <LiquidCrystal_I2C.h>

#include <Wire.h>

// =========================

// WIFI CONFIGURATION

// =========================

const char* ssid = "Monchel";

const char* password = "Monchel1236";

// REPLACE "YOUR_COMPUTER_IP_ADDRESS" with the local IP of the PC running the backend

const char* backendUrl = "http://192.168.202.125:5000/api/telemetry";

// =========================

// DHT CONFIGURATION

// =========================

#define DHTTYPE DHT11

#define DHT_ROOM1_PIN 4

#define DHT_ROOM2_PIN 5

DHT dhtRoom1(DHT_ROOM1_PIN, DHTTYPE);

DHT dhtRoom2(DHT_ROOM2_PIN, DHTTYPE);

// =========================

// RELAY PINS

// =========================

#define COOL_FAN_ROOM1 18

#define COOL_FAN_ROOM2 19

#define WARM_FAN_ROOM2 21

// =========================

// LED PINS

// =========================

#define RED_LED 22

#define GREEN_LED 23

// =========================

// MQ135 PIN

// =========================

#define MQ135_PIN 34

// =========================

// LCD CONFIGURATION

// =========================

LiquidCrystal_I2C lcd(0x27, 16, 2);

// Target Temps

float room1Min = 2.0;

float room1Max = 8.0;

float room2Min = 0.0;

float room2Max = 2.0;

// Manual Controls from Web

bool r1Sys = true;

bool r1Fan = false;

bool r1Comp = true;

bool r2Sys = true;

bool r2Fan = false;

bool r2Comp = true;

// =========================

// VARIABLES

// =========================

float realTemp1, hum1;

float realTemp2, hum2;

float temp1;

float temp2;

int airQuality;

String alertMessage = "SAFE";

// =========================

// SETUP

// =========================

void setup() {

Serial.begin(115200);

// Start WiFi

Serial.print("Connecting to WiFi");

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nConnected to WiFi");

Serial.print("ESP32 IP: ");

Serial.println(WiFi.localIP());

// Start DHT Sensors

dhtRoom1.begin();

dhtRoom2.begin();

// Relay Pins

pinMode(COOL_FAN_ROOM1, OUTPUT);

pinMode(COOL_FAN_ROOM2, OUTPUT);

pinMode(WARM_FAN_ROOM2, OUTPUT);

// LED Pins

pinMode(RED_LED, OUTPUT);

pinMode(GREEN_LED, OUTPUT);

// MQ135

pinMode(MQ135_PIN, INPUT);

// LCD I2C

Wire.begin(26, 27);

lcd.init();

lcd.backlight();

// Initial Relay States

digitalWrite(COOL_FAN_ROOM1, LOW);

digitalWrite(COOL_FAN_ROOM2, LOW);

digitalWrite(WARM_FAN_ROOM2, LOW);

// Welcome Screen

lcd.setCursor(0, 0);

lcd.print("ColdChain");

lcd.setCursor(0, 1);

lcd.print("Sentinel");

delay(3000);

lcd.clear();

}

void sendTelemetryToBackend(float realT1, float realH1, float realT2, float realH2, float t1, float h1, float t2, float h2, int aqi) {

if(WiFi.status() == WL_CONNECTED){

HTTPClient http;

http.begin(backendUrl);

http.addHeader("Content-Type", "application/json");

// Average of both DHT sensors = ambient room conditions

float avgTemp = (realT1 + realT2) / 2.0;

float avgHum  = (realH1 + realH2) / 2.0;

// Build JSON payload

// room.ip        → matches StorageRoom.ipAddress in DB (192.168.1.20)

// fridges[0].ip  → matches Device.ipAddress for Main Fridge (192.168.1.10)

// fridges[1].ip  → matches Device.ipAddress for Vaccine Freezer (192.168.1.11)

String payload = "{";

payload += "\"secret\":\"supersecretkey123\",";

payload += "\"room\":{";

payload += "\"ip\":\"192.168.1.20\",";

payload += "\"airQuality\":" + String(aqi) + ",";

payload += "\"ambientTemp\":" + String(avgTemp, 1) + ",";

payload += "\"ambientHum\":"  + String(avgHum,  1);

payload += "},";

payload += "\"fridges\":[";

payload += "{\"ip\":\"192.168.1.10\",\"temperature\":" + String(t1, 1) + ",\"humidity\":" + String(h1, 1) + "},";

payload += "{\"ip\":\"192.168.1.11\",\"temperature\":" + String(t2, 1) + ",\"humidity\":" + String(h2, 1) + "}";

payload += "]}";

Serial.println("Sending telemetry to backend...");

int httpResponseCode = http.POST(payload);

if(httpResponseCode > 0){

Serial.print("Telemetry HTTP response code: ");

Serial.println(httpResponseCode);

String response = http.getString();

Serial.print("Backend response: ");

Serial.println(response);

// Parse JSON from Backend

DynamicJsonDocument doc(1024);

DeserializationError error = deserializeJson(doc, response);

if (!error) {

Serial.println("Telemetry sent successfully and response parsed.");

if (doc["config"]) {

Serial.println("Backend returned configuration updates.");

if (doc["config"]["192.168.1.10"]) {

room1Min = doc["config"]["192.168.1.10"]["min"];

room1Max = doc["config"]["192.168.1.10"]["max"];

r1Sys    = doc["config"]["192.168.1.10"]["sys"];

r1Fan    = doc["config"]["192.168.1.10"]["fan"];

r1Comp   = doc["config"]["192.168.1.10"]["comp"];

}

if (doc["config"]["192.168.1.11"]) {

room2Min = doc["config"]["192.168.1.11"]["min"];

room2Max = doc["config"]["192.168.1.11"]["max"];

r2Sys    = doc["config"]["192.168.1.11"]["sys"];

r2Fan    = doc["config"]["192.168.1.11"]["fan"];

r2Comp   = doc["config"]["192.168.1.11"]["comp"];

}

} else {

Serial.println("No config object returned by backend.");

}

} else {

Serial.print("Failed to parse backend response: ");

Serial.println(error.c_str());

}

} else {

Serial.print("Error sending to backend: ");

Serial.println(httpResponseCode);

}

http.end();

} else {

Serial.println("Error in WiFi connection");

}

}

// =========================

// LOOP

// =========================

void loop() {

// =========================

// READ REAL SENSOR VALUES

// =========================

realTemp1 = dhtRoom1.readTemperature();

hum1 = dhtRoom1.readHumidity();

realTemp2 = dhtRoom2.readTemperature();

hum2 = dhtRoom2.readHumidity();

// =========================

// SIMULATE FRIDGE TEMPERATURES

// Convert:

// 24°C - 28°C  -->  2°C - 8°C

// =========================

temp1 = (realTemp1 - 24.0) * (8.0 - 2.0) / (28.0 - 24.0) + 2.0;

// Convert:

// 24°C - 28°C  -->  0°C - 2°C

temp2 = (realTemp2 - 24.0) * (2.0 - 0.0) / (28.0 - 24.0) + 0.0;

// =========================

// READ AIR QUALITY

// =========================

// airQuality = analogRead(MQ135_PIN);

airQuality = 302;

// =========================

// SERIAL MONITOR

// =========================

Serial.println("================================");

Serial.print("Real Temp Room1: ");

Serial.print(realTemp1);

Serial.println(" C");

Serial.print("Simulated Room1 Temp: ");

Serial.print(temp1);

Serial.println(" C");

Serial.print("Room1 Humidity: ");

Serial.print(hum1);

Serial.println(" %");

Serial.println("--------------------------------");

Serial.print("Real Temp Room2: ");

Serial.print(realTemp2);

Serial.println(" C");

Serial.print("Simulated Room2 Temp: ");

Serial.print(temp2);

Serial.println(" C");

Serial.print("Room2 Humidity: ");

Serial.print(hum2);

Serial.println(" %");

Serial.println("--------------------------------");

Serial.print("Air Quality: ");

Serial.println(airQuality);

// =========================

// ROOM 1 CONTROL (Refrigerator)

// =========================

if (!r1Sys) {

digitalWrite(COOL_FAN_ROOM1, LOW); // System off

} else {

// If manual Fan is ON, or (Compressor is ON and temp is too high)

if (r1Fan || (r1Comp && temp1 > room1Max)) {

digitalWrite(COOL_FAN_ROOM1, HIGH);

} else {

digitalWrite(COOL_FAN_ROOM1, LOW);

}

}

// =========================

// ROOM 2 CONTROL (Freezer)

// =========================

if (!r2Sys) {

digitalWrite(COOL_FAN_ROOM2, LOW);

digitalWrite(WARM_FAN_ROOM2, LOW);

} else {

bool coolOn = r2Fan; // Fan switch forces circulation (cooling fan)

bool warmOn = false;

if (r2Comp) {

if (temp2 > room2Max) {

coolOn = true;

warmOn = false;

} else if (temp2 < room2Min) {

coolOn = false;

warmOn = true;

}

}

digitalWrite(COOL_FAN_ROOM2, coolOn ? HIGH : LOW);

digitalWrite(WARM_FAN_ROOM2, warmOn ? HIGH : LOW);

}

// =========================

// ALERT DETECTION

// =========================

bool danger = false;

alertMessage = "SAFE";

// ROOM 1 ALERTS

if (temp1 > room1Max) {

danger = true;

alertMessage = "R1 HIGH TEMP";

}

else if (temp1 < room1Min) {

danger = true;

alertMessage = "R1 LOW TEMP";

}

// ROOM 2 ALERTS

else if (temp2 > room2Max) {

danger = true;

alertMessage = "R2 HIGH TEMP";

}

else if (temp2 < room2Min) {

danger = true;

alertMessage = "R2 LOW TEMP";

}

// AIR QUALITY ALERT

else if (airQuality > 2500) {

danger = true;

alertMessage = "BAD AIR";

}

if (!r1Sys && !r2Sys) {

alertMessage = "SYS STOPPED";

danger = false; // Override danger LED if purposefully stopped

}

// =========================

// LED STATUS

// =========================

if (danger) {

digitalWrite(RED_LED, HIGH);

digitalWrite(GREEN_LED, LOW);

}

else {

digitalWrite(RED_LED, LOW);

digitalWrite(GREEN_LED, HIGH);

}

// =========================

// PRINT ALERT STATUS

// =========================

Serial.print("System Status: ");

Serial.println(alertMessage);

// =========================

// LCD DISPLAY — 4 screens, 4 seconds each

// =========================

if (!r1Sys && !r2Sys) {

// ── System stopped ──────────────────────────

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("SYSTEM STOPPED");

lcd.setCursor(0, 1);

lcd.print("All units OFF");

delay(4000);

} else {

// ── Screen 1: Fridge 1 temp & humidity ──────

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Fridge 1");

if (!r1Sys) {

lcd.setCursor(0, 1);

lcd.print("System OFF");

} else {

lcd.setCursor(0, 1);

lcd.print("T:");

lcd.print(temp1, 1);

lcd.print("C H:");

lcd.print(hum1, 0);

lcd.print("%");

}

delay(4000);

// ── Screen 2: Fridge 2 temp & humidity ──────

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Fridge 2");

if (!r2Sys) {

lcd.setCursor(0, 1);

lcd.print("System OFF");

} else {

lcd.setCursor(0, 1);

lcd.print("T:");

lcd.print(temp2, 1);

lcd.print("C H:");

lcd.print(hum2, 0);

lcd.print("%");

}

delay(4000);

// ── Screen 3: Air quality (MQ135 raw ADC) ───

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Air Quality");

lcd.setCursor(0, 1);

lcd.print("MQ135: ");

lcd.print(airQuality);

delay(4000);

// ── Screen 4: System status / alert ─────────

lcd.clear();

lcd.setCursor(0, 0);

if (danger) {

lcd.print("!! ALERT !!");

} else {

lcd.print("Status: OK");

}

lcd.setCursor(0, 1);

lcd.print(alertMessage);

delay(4000);

}

// SEND REAL-TIME DATA TO DASHBOARD

sendTelemetryToBackend(realTemp1, hum1, realTemp2, hum2, temp1, hum1, temp2, hum2, airQuality);

}